COMMANDS:

ROUTER1:

enable

configure terminal

hostname R1

interface g0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface s0/0/0

ip address 10.10.10.1 255.255.255.252

clock rate 64000

no shutdown

exit

ip route 192.168.2.0 255.255.255.0 10.10.10.2

exit

ROUTER2:

enable

configure terminal

hostname R2

interface g0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface s0/0/0

ip address 10.10.10.2 255.255.255.252

no shutdown

exit

ip route 192.168.1.0 255.255.255.0 10.10.10.1

exit

crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac

mode tunnel

exit

ROUTER1:

crypto isakmp policy 10

encryption aes

hash sha

authentication pre-share

group 2

lifetime 86400

exit

crypto isakmp key cisco123 address 10.10.10.2

crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac

access-list 100 permit ip 192.168.1.0 0.0.0.255 192.168.2.0 0.0.0.255

crypto map VPN-MAP 10 ipsec-isakmp

set peer 10.10.10.2

set transform-set VPN-SET

match address 100

exit

interface fa0/0

crypto map VPN-MAP

exit

exit

wr

ROUTER2:

crypto isakmp policy 10

encryption aes

hash sha

authentication pre-share

group 2

lifetime 86400

exit

crypto isakmp key cisco123 address 10.10.10.1

crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac

access-list 100 permit ip 192.168.2.0 0.0.0.255 192.168.1.0 0.0.0.255

crypto map VPN-MAP 10 ipsec-isakmp

set peer 10.10.10.1

set transform-set VPN-SET

match address 100

exit

interface fa0/0

crypto map VPN-MAP

exit

exit

wr

OUTPUT: